PART II  —  C17 CONSTRUCTS

CHAPTER 6

# Structures, Alignment, and Hardware Mapping

A struct in C is not merely a grouping of values. It is a blueprint for memory: a precise specification of how bytes are arranged, how the hardware will interpret them, and how the compiler may transform them. In embedded and safety-critical systems, the gap between the programmer's mental model and the compiler's actual layout has caused defects ranging from wrong sensor readings to complete system failures. — Author's Note

The struct is the fundamental unit of data organization in C. Nearly every data structure in a safety-critical system — sensor readings, CAN message frames, hardware register maps, state machines, protocol headers — is expressed as a struct. Yet for all its ubiquity, the struct is one of the most frequently misunderstood constructs in the language.

The misunderstanding centers on layout. Most programmers intuitively model a struct as a contiguous sequence of its member values, packed tightly together. The actual layout includes compiler-inserted padding bytes whose size and position follow rules that depend on the type, size, and order of members, the target architecture, and the compilation flags. The difference between the mental model and the actual layout is the source of bugs in hardware register mapping, inter-process communication, network protocol handling, and data serialization.

This chapter builds a precise, complete understanding of struct layout in C17. It covers the alignment algorithm, the rules for padding insertion, the tools for verifying layout, and the patterns for hardware register mapping. It examines bit-fields, struct packing, designated initializers, flexible array members, and nested structures. Throughout, it connects each concept to safety-critical system design and LLM infrastructure engineering.

Originality declaration: all prose, diagrams, code examples, analysis, and tables in this chapter are composed specifically for this work. No content is reproduced from any external source.

## 6.1  How the Compiler Lays Out Structs

To understand struct layout, we must understand alignment. Every object type in C has an alignment requirement: the addresses at which objects of that type may be placed must be multiples of the type's alignment value. On typical 32-bit and 64-bit platforms:

| Type | Size (bytes) | Alignment (bytes) | Notes |
| --- | --- | --- | --- |
| char / uint8_t | 1 | 1 | May be placed at any address |
| short / uint16_t | 2 | 2 | Must be at even address |
| int / uint32_t | 4 | 4 | Must be at address divisible by 4 |
| long / uint64_t (LP64) | 8 | 8 | Must be at address divisible by 8 |
| float | 4 | 4 | Same as uint32_t |
| double | 8 | 8 | Same as uint64_t on most platforms |
| pointer (64-bit) | 8 | 8 | Same as uint64_t |
| struct | max of member aligns | max of member aligns | Struct size is multiple of its alignment |

The compiler lays out struct members in declaration order, inserting padding bytes as needed to satisfy each member's alignment requirement. It also adds trailing padding at the end of the struct so that the struct's total size is a multiple of its largest member's alignment. This trailing padding ensures that arrays of structs are correctly aligned: if sizeof(S) were not a multiple of S's alignment, the second element of S[] would be misaligned.

### 6.1.1  The Padding Algorithm: Step by Step

The algorithm the compiler uses to lay out a struct can be stated precisely. Starting with offset 0, for each member in declaration order: advance the current offset to the next multiple of the member's alignment requirement, place the member at that offset, advance the current offset by the member's size. After all members are placed, advance the total size to the next multiple of the struct's overall alignment (the maximum alignment of any member).

struct Example { Offset Size Align Notes uint8_t a; -> 0 1 1 no padding needed // 3 bytes padding -> 1 3 - pad to align 'b' uint32_t b; -> 4 4 4 aligned at 4 uint16_t c; -> 8 2 2 aligned at 8 (ok) // 2 bytes padding -> 10 2 - pad struct to mult of 4 uint8_t d; -> 10 1 1 (note: after c) // 1 byte padding -> 11 1 - pad to mult of 4 }; sizeof = 12 alignof = 4 Wait: let's redo correctly with the right offsets: uint8_t a -> offset 0, size 1 [3 bytes padding] -> offsets 1,2,3 uint32_t b -> offset 4, size 4 uint16_t c -> offset 8, size 2 uint8_t d -> offset 10, size 1 [1 byte padding] -> offset 11 sizeof(Example) = 12 alignof(Example) = 4

Listing 6.1 — struct layout analysis with static_assert verification

#include <stdint.h> #include <stddef.h> #include <assert.h> // ── Analyze and document layout of a typical sensor record ──── typedef struct { uint8_t sensor_id; // offset 0, size 1, align 1 // PADDING: 3 bytes // offset 1..3 uint32_t timestamp_ms; // offset 4, size 4, align 4 float temperature; // offset 8, size 4, align 4 uint16_t sequence; // offset 12, size 2, align 2 uint8_t status; // offset 14, size 1, align 1 // PADDING: 1 byte // offset 15 // Total: 16 bytes. Largest member alignment: 4. // sizeof = 16 (multiple of 4). Correct. } SensorRecord; // Always verify layout with static_assert. // These assertions serve as documentation AND compile-time checks. // If the platform changes, compilation fails immediately. static_assert(sizeof(SensorRecord) == 16, "SensorRecord must be 16 bytes"); static_assert(offsetof(SensorRecord, sensor_id) == 0, "sensor_id must be at offset 0"); static_assert(offsetof(SensorRecord, timestamp_ms) == 4, "timestamp_ms must be at offset 4"); static_assert(offsetof(SensorRecord, temperature) == 8, "temperature must be at offset 8"); static_assert(offsetof(SensorRecord, sequence) == 12, "sequence must be at offset 12"); static_assert(offsetof(SensorRecord, status) == 14, "status must be at offset 14"); // ── Reordering members to minimize padding ──────────────────── // Original layout wastes 4 bytes padding. // Reordered by descending alignment: no padding, smaller size. typedef struct { uint32_t timestamp_ms; // offset 0, size 4, align 4 float temperature; // offset 4, size 4, align 4 uint16_t sequence; // offset 8, size 2, align 2 uint8_t sensor_id; // offset 10, size 1, align 1 uint8_t status; // offset 11, size 1, align 1 // No padding! sizeof = 12. } SensorRecordPacked; // misleading name: no __packed, just reordered static_assert(sizeof(SensorRecordPacked) == 12, "Reordered SensorRecord must be 12 bytes (no padding)"); // Rule of thumb: declare members largest-to-smallest alignment // to minimize compiler-inserted padding. // Exception: when struct maps to hardware register layout, // member order is dictated by the hardware spec, not optimization.

### 6.1.2  Alignment of Composite Types

The alignment requirement of a struct is the maximum alignment requirement of any of its members. This is because the struct as a whole must be aligned so that every member is aligned: if the struct is placed at an address that is a multiple of its own alignment, and member offsets are multiples of member alignments, then all members will be aligned.

This rule has an important consequence for nested structs: a struct containing a double (alignment 8) has alignment 8, even if the double is the only member with alignment 8. Placing the outer struct at any address that is not a multiple of 8 would misalign the double member.

Listing 6.2 — alignment of nested and composite types

#include <stdint.h> #include <stddef.h> #include <stdalign.h> // Alignment of a struct equals the maximum member alignment. typedef struct { uint8_t byte_field; // align 1 double double_field; // align 8: drives struct alignment uint32_t word_field; // align 4 } MixedAlign; // alignof(MixedAlign) == 8 (due to double_field) static_assert(alignof(MixedAlign) == 8, "MixedAlign alignment must be 8 due to double member"); // Layout: // byte_field: offset 0, size 1 // [7 padding]: offset 1..7 // double_field: offset 8, size 8 // word_field: offset 16, size 4 // [4 padding]: offset 20..23 // sizeof = 24 static_assert(sizeof(MixedAlign) == 24, "must be 24"); static_assert(offsetof(MixedAlign, double_field) == 8, "double must be at offset 8"); // Nested struct: inherits the worst alignment of its members. typedef struct { uint16_t header; // align 2 MixedAlign payload; // align 8: drives outer struct alignment uint8_t checksum; // align 1 } OuterStruct; // alignof(OuterStruct) == 8 (due to MixedAlign payload) static_assert(alignof(OuterStruct) == 8, "OuterStruct alignment must be 8"); // Layout: // header: offset 0, size 2 // [6 padding] offset 2..7 // payload: offset 8, size 24 // checksum: offset 32, size 1 // [7 padding] offset 33..39 // sizeof = 40 static_assert(sizeof(OuterStruct) == 40, "must be 40"); static_assert(offsetof(OuterStruct, payload) == 8, "payload at offset 8"); static_assert(offsetof(OuterStruct, checksum) == 32, "checksum at offset 32");

## 6.2  sizeof, offsetof, and Static Assertion Discipline

The two most important tools for documenting and verifying struct layout are sizeof and offsetof. sizeof returns the total size of a type in bytes, including all padding. offsetof returns the byte offset of a named member within a struct. Together, they provide a complete description of layout that can be verified at compile time using static_assert.

In safety-critical systems, using static_assert to document and verify struct layout is not optional for hardware-mapped structures. Every struct that maps to a hardware register, protocol frame, or inter-process communication buffer must have its layout verified at compile time. A layout that changes due to a compiler version upgrade or platform change must be caught at compile time, not at runtime.

### 6.2.1  The offsetof Macro: Correct and Incorrect Use

offsetof(type, member) is defined in <stddef.h>. It expands to a constant expression of type size_t equal to the offset in bytes of the named member from the beginning of the struct. It is defined for any struct member that is not a bit-field.

A common misuse of offsetof is to compute it at runtime using pointer arithmetic. The macro form is guaranteed to produce a compile-time constant, which is required for use in static_assert. The pointer arithmetic form ((size_t)&((T*)0)->member) is technically undefined behavior in C11 (dereferencing a null pointer is UB even when the result is unused), even though it works in practice on all common platforms.

Listing 6.3 — offsetof: correct use and common misuse

#include <stddef.h> #include <stdint.h> #include <assert.h> typedef struct { uint32_t id; uint16_t value; uint8_t flags; uint8_t checksum; } CanFrame; // ── CORRECT: compile-time offsetof via macro ────────────────── // offsetof() produces a constant expression. Use in static_assert. static_assert(offsetof(CanFrame, id) == 0, "id at 0"); static_assert(offsetof(CanFrame, value) == 4, "value at 4"); static_assert(offsetof(CanFrame, flags) == 6, "flags at 6"); static_assert(offsetof(CanFrame, checksum) == 7, "checksum at 7"); static_assert(sizeof(CanFrame) == 8, "CanFrame must be 8 bytes"); // ── CORRECT: runtime field access via offsetof ──────────────── // Useful for generic serialization functions that operate on // field descriptors rather than explicit member access. typedef struct { const char *name; size_t offset; size_t size; } FieldDescriptor; static const FieldDescriptor CAN_FRAME_FIELDS[] = { {"id", offsetof(CanFrame, id), sizeof(uint32_t)}, {"value", offsetof(CanFrame, value), sizeof(uint16_t)}, {"flags", offsetof(CanFrame, flags), sizeof(uint8_t)}, {"checksum", offsetof(CanFrame, checksum), sizeof(uint8_t)}, }; #define CAN_FRAME_FIELD_COUNT \ (sizeof(CAN_FRAME_FIELDS) / sizeof(CAN_FRAME_FIELDS[0])) // Generic serializer: writes struct fields to byte buffer. // Works for any struct with a field descriptor table. void serialize_struct(const void *src, uint8_t *dst, size_t dst_size, const FieldDescriptor *fields, uint32_t field_count) { size_t write_pos = 0U; for (uint32_t i = 0U; i < field_count; i++) { if (write_pos + fields[i].size > dst_size) break; // Copy field bytes from struct to output buffer. // Note: no padding bytes included; only value bytes. const uint8_t *src_field = (const uint8_t *)src + fields[i].offset; for (size_t j = 0U; j < fields[i].size; j++) { dst[write_pos++] = src_field[j]; } } } // ── INCORRECT: pointer arithmetic form (UB in C11) ─────────── // This works in practice but is technically UB. // Never use in safety-critical code; use offsetof macro instead. #define OFFSET_WRONG(type, member) \ ((size_t)&((type *)0)->member) // UB: null pointer deref

## 6.3  Hardware Register Mapping: Memory-Mapped I/O Struct Patterns

Memory-mapped I/O (MMIO) is the mechanism by which embedded software communicates with hardware peripherals. Hardware registers are accessed by reading and writing specific memory addresses. Using structs to map these registers provides a clean, named interface while ensuring that the correct bytes are accessed at the correct addresses.

MMIO struct mapping is one of the most common uses of structs in embedded systems — and one of the most error-prone. The struct must map exactly to the hardware's register layout: wrong padding, wrong types, or wrong member order will cause incorrect register accesses that may be silent or intermittent.

### 6.3.1  The MMIO Struct Pattern

The canonical pattern for MMIO struct mapping uses a typedef to define the register layout, __attribute__((packed)) or explicit padding to match the hardware layout, volatile to ensure reads and writes reach the hardware, and a macro or constant pointer to bind the struct to the hardware base address.

Listing 6.4 — MMIO struct pattern: UART controller mapping

#include <stdint.h> #include <stddef.h> // ── UART peripheral register layout ─────────────────────────── // Based on a typical ARM Cortex-M UART peripheral. // All values are from the hardware reference manual. // Every field documented: name, offset, width, purpose. typedef struct { volatile uint32_t DR; // offset 0x00: Data Register // [7:0] data, [11:8] error flags volatile uint32_t RSR_ECR; // offset 0x04: Receive Status / Error Clear volatile uint32_t RESERVED0[4]; // offset 0x08..0x14: reserved volatile uint32_t FR; // offset 0x18: Flag Register // [0]=CTS, [3]=BUSY, [4]=RXFE, // [5]=TXFF, [6]=RXFF, [7]=TXFE volatile uint32_t RESERVED1; // offset 0x1C: reserved volatile uint32_t ILPR; // offset 0x20: IrDA Low-Power Counter volatile uint32_t IBRD; // offset 0x24: Integer Baud Rate Divisor volatile uint32_t FBRD; // offset 0x28: Fractional Baud Rate Divisor volatile uint32_t LCRH; // offset 0x2C: Line Control Register volatile uint32_t CR; // offset 0x30: Control Register volatile uint32_t IFLS; // offset 0x34: Interrupt FIFO Level Select volatile uint32_t IMSC; // offset 0x38: Interrupt Mask Set/Clear volatile uint32_t RIS; // offset 0x3C: Raw Interrupt Status volatile uint32_t MIS; // offset 0x40: Masked Interrupt Status volatile uint32_t ICR; // offset 0x44: Interrupt Clear Register volatile uint32_t DMACR; // offset 0x48: DMA Control Register } UART_TypeDef; // ── Verify layout at compile time ───────────────────────────── static_assert(offsetof(UART_TypeDef, DR) == 0x00U, "DR at 0x00"); static_assert(offsetof(UART_TypeDef, FR) == 0x18U, "FR at 0x18"); static_assert(offsetof(UART_TypeDef, IBRD) == 0x24U, "IBRD at 0x24"); static_assert(offsetof(UART_TypeDef, CR) == 0x30U, "CR at 0x30"); static_assert(offsetof(UART_TypeDef, ICR) == 0x44U, "ICR at 0x44"); // ── Base address binding ────────────────────────────────────── #define UART0_BASE 0x40011000UL #define UART1_BASE 0x40011400UL #define UART0 ((UART_TypeDef *)UART0_BASE) #define UART1 ((UART_TypeDef *)UART1_BASE) // ── Register bit definitions ────────────────────────────────── #define UART_FR_RXFE (1U << 4) // Receive FIFO empty #define UART_FR_TXFF (1U << 5) // Transmit FIFO full #define UART_FR_RXFF (1U << 6) // Receive FIFO full #define UART_FR_TXFE (1U << 7) // Transmit FIFO empty #define UART_CR_UARTEN (1U << 0) // UART enable #define UART_CR_TXE (1U << 8) // Transmit enable #define UART_CR_RXE (1U << 9) // Receive enable // ── Usage: clean, readable hardware access ──────────────────── void uart_send_byte(UART_TypeDef *uart, uint8_t byte) { // Wait until TX FIFO is not full while (uart->FR & UART_FR_TXFF) { } uart->DR = byte; } uint8_t uart_recv_byte(UART_TypeDef *uart) { // Wait until RX FIFO has data while (uart->FR & UART_FR_RXFE) { } return (uint8_t)(uart->DR & 0xFFU); } void uart_init(UART_TypeDef *uart, uint32_t baud_divisor_int, uint32_t baud_divisor_frac) { uart->CR = 0U; // disable first uart->IBRD = baud_divisor_int; uart->FBRD = baud_divisor_frac; uart->LCRH = (3U << 5); // 8-bit, no parity uart->CR = UART_CR_UARTEN ¦ UART_CR_TXE ¦ UART_CR_RXE; }

🔌 MMIO Struct Safety Rules All members of an MMIO struct must be declared volatile. A non-volatile member will have its accesses optimized, potentially caching register values in CPU registers and never reading the hardware. Reserved registers must be included as padding members (uint32_t RESERVED[n]) to maintain correct offsets for subsequent registers. Never skip reserved regions. Always verify layout with static_assert on every offset and on the total struct size. Hardware reference manuals sometimes have errata; the static_assert catches the discrepancy immediately. The MMIO struct pointer must be defined as a compile-time constant pointer. Never compute the base address at runtime; runtime computation errors are silent.

### 6.3.2  Read-Write, Read-Only, and Write-Only Registers

Many hardware registers are not bidirectional. Some are read-only (hardware writes, software reads): reading them returns a status value, writing has no defined effect. Others are write-only (software writes, hardware reads): writing them triggers an action, but reading returns an undefined value. Modeling these distinctions in the struct type catches misuse at compile time.

Listing 6.5 — read-only and write-only register qualification

#include <stdint.h> // ── Read-only register: const volatile ──────────────────────── // 'const' prevents software writes. // 'volatile' ensures reads reach the hardware. typedef struct { volatile const uint32_t STATUS; // RO: hardware writes volatile uint32_t CONTROL; // RW: both directions volatile uint32_t COMMAND; // WO (conceptually) volatile const uint32_t ID; // RO: chip identifier } PeripheralRegs; // Attempting to write a const volatile register: compile error. // UART0->STATUS = 0; // ERROR: assignment of read-only member // ── Write-only simulation: separate read/write types ────────── // C does not have a 'write-only' qualifier, but we can enforce // the intent by providing a write-only wrapper that does not // expose the read interface. typedef struct { void (*write)(uint32_t value); // only write supported } WriteOnlyReg; // Practical approach: document write-only registers in comments // and use a dedicated write-only accessor function. #define CMD_REG_ADDR 0x40012010UL // Write-only: do not read. Reading returns undefined value. static inline void cmd_reg_write(uint32_t cmd) { *((volatile uint32_t *)CMD_REG_ADDR) = cmd; } // No cmd_reg_read() function provided: enforces write-only intent. // ── Clear-on-read registers ──────────────────────────────────── // Some status registers clear their bits when read. // Reading them twice gives different results. // Document clearly; read exactly once and save the value. uint32_t read_interrupt_status(PeripheralRegs *regs) { // Read ONCE: the read clears the interrupt flags. // Storing in a local variable before any conditional checks // ensures we do not lose flags by re-reading. uint32_t status = regs->STATUS; // single read return status; // Never: if (regs->STATUS & FLAG_A) { ... } // if (regs->STATUS & FLAG_B) { ... } // Second read may return 0 (flags already cleared by first read). }

## 6.4  Bit-Fields: Safe Use and Portability Hazards

Bit-fields allow struct members to occupy fewer bits than a full byte. They are frequently used in embedded systems to map hardware register bit fields, implement compact protocol headers, and pack boolean flags into a single word. However, bit-fields are among the most implementation-defined constructs in C, with behavior that varies between compilers, architectures, and even compilation modes.

### 6.4.1  What the Standard Guarantees

C17 specifies the following about bit-fields: a bit-field may have any width from 1 to the number of bits in its underlying type; a bit-field of width 0 forces alignment to the next unit boundary; unnamed bit-fields may be used as padding; the order of bit-field allocation within a storage unit (from the most significant bit or least significant bit) is implementation-defined; a bit-field may not span multiple storage units in some implementations.

This leaves an enormous amount of behavior unspecified. The practical consequence: bit-field layouts are not portable across compilers, architectures, or even between big-endian and little-endian targets of the same architecture.

Listing 6.6 — bit-fields: what is defined, what is not

#include <stdint.h> #include <stddef.h> // ── Bit-field layout is IMPLEMENTATION-DEFINED ──────────────── // On little-endian x86-64 with GCC/Clang: // status_bit occupies bit 0 (LSB) of the storage unit. // error_code occupies bits 1..4. // reserved occupies bits 5..7. // On big-endian PowerPC with GCC: // status_bit occupies bit 7 (MSB) of the storage unit. // The entire layout is REVERSED. // This is NOT portable. Never use bit-field structs for // cross-platform or cross-compiler data exchange. typedef struct { uint8_t status_bit : 1; // 1-bit field uint8_t error_code : 4; // 4-bit field uint8_t reserved : 3; // 3-bit padding } StatusByte; // sizeof(StatusByte) == 1 on most platforms (fits in one byte) // But the BIT ORDER within that byte is implementation-defined. // ── SAFE use: flags within a single word, same platform ─────── // Using bit-fields for software-internal flags (not hardware maps) // is safer because portability is less critical. typedef struct { unsigned int initialized : 1; unsigned int connected : 1; unsigned int error : 1; unsigned int busy : 1; unsigned int : 28; // unnamed: padding to 32 bits } DeviceFlags; // ── UNSAFE: using bit-fields to map hardware registers ───────── // The compiler may lay out bits in any order. // If the hardware manual says bit 0 is the enable bit, // your bit-field may or may not map to bit 0. typedef struct { uint32_t enable : 1; // UNSAFE: may not be bit 0 on all targets uint32_t mode : 3; // UNSAFE: may not be bits 1..3 uint32_t divider : 8; // UNSAFE: may not be bits 4..11 uint32_t : 20; // padding } HwControlReg_UNSAFE; // ── SAFE: use explicit masks and shifts for hardware registers ─ // This approach is fully portable and unambiguous. #define HW_CTRL_ENABLE_BIT (1U << 0) #define HW_CTRL_MODE_SHIFT 1U #define HW_CTRL_MODE_MASK (0x7U << HW_CTRL_MODE_SHIFT) #define HW_CTRL_DIVIDER_SHIFT 4U #define HW_CTRL_DIVIDER_MASK (0xFFU << HW_CTRL_DIVIDER_SHIFT) // Reading and writing fields with masks and shifts: void set_hw_mode(volatile uint32_t *ctrl_reg, uint32_t mode) { uint32_t val = *ctrl_reg; val &= ~HW_CTRL_MODE_MASK; // clear mode field val ¦= (mode << HW_CTRL_MODE_SHIFT) & HW_CTRL_MODE_MASK; // set new mode *ctrl_reg = val; } uint32_t get_hw_divider(volatile const uint32_t *ctrl_reg) { return (*ctrl_reg & HW_CTRL_DIVIDER_MASK) >> HW_CTRL_DIVIDER_SHIFT; }

📘 MISRA — MISRA C:2012 Rule 6.1 — Required Bit-fields shall only be declared with an explicitly unsigned integer type or a specifically signed integer type. Using 'int' or 'signed int' for bit-fields is prohibited because whether a plain int bit-field is signed or unsigned is implementation-defined. Use 'uint8_t : n', 'uint16_t : n', or 'uint32_t : n' for bit-fields. Never use plain 'int : n', 'char : n', or enumeration types for bit-fields.

📘 MISRA — MISRA C:2012 Rule 6.2 — Required Single-bit named bit-fields shall not be of a signed type. A signed single-bit bit-field can only represent the values 0 and -1 (in two's complement). This is almost certainly not the programmer's intent and is a common source of sign comparison bugs. Use unsigned types for all single-bit flags: 'uint8_t flag : 1', never 'int flag : 1'.

## 6.5  Struct Packing: __attribute__((packed)) and Its Risks

Struct packing is the technique of instructing the compiler to eliminate padding from a struct, producing the smallest possible layout. In GCC and Clang, this is done with __attribute__((packed)). On MSVC, the equivalent is #pragma pack(1). Packed structs are useful for: serializing data to network or file formats, mapping hardware protocols with non-aligned fields, and reducing memory usage in memory-constrained systems.

However, packed structs come with serious risks on platforms that require aligned memory access. Understanding these risks is essential before using __attribute__((packed)) in any safety-critical system.

### 6.5.1  Why Packing Works on x86 and Fails on ARM

x86 and x86-64 processors support unaligned memory access in hardware. When the CPU encounters a load or store to a misaligned address, it performs the access correctly, usually with a small performance penalty. As a result, packed structs work correctly on x86, though with reduced performance for misaligned accesses.

ARM Cortex-M processors (used in most automotive and industrial microcontrollers) do not support unaligned access by default. Attempting to read a uint32_t from an address that is not a multiple of 4 raises a HardFault exception. On ARMv7-A and ARMv8-A application processors, unaligned access is supported but may be slow and may violate atomicity requirements. On some DSP and RISC-V platforms, unaligned access always faults.

Listing 6.7 — packed struct risks: x86 vs ARM

#include <stdint.h> #include <string.h> // Packed struct: no padding. Smallest possible layout. typedef struct __attribute__((packed)) { uint8_t type; // offset 0, size 1 uint32_t timestamp; // offset 1, size 4 <- MISALIGNED! Not at mult of 4 uint16_t length; // offset 5, size 2 <- MISALIGNED! Not at mult of 2 uint8_t payload[8]; // offset 7, size 8 } PackedProtocolFrame; // sizeof = 15 (no padding) // ── On x86-64: works but slower ─────────────────────────────── // The CPU handles misaligned access in hardware. // Performance penalty: ~2x slower than aligned access. // Correctness: correct. // ── On ARM Cortex-M (e.g., STM32): HARDFAULT ────────────────── // Accessing frame->timestamp generates a LDR to address+1. // If CCR.UNALIGN_TRP is set (default in some configs): HardFault. // If not set: hardware may silently return wrong value. void process_frame_UNSAFE(const PackedProtocolFrame *frame) { uint32_t ts = frame->timestamp; // UNSAFE: misaligned on ARM (void)ts; } // ── SAFE: use memcpy to extract misaligned fields ────────────── // memcpy handles any alignment correctly on all platforms. // GCC/Clang optimize this to efficient load instructions. void process_frame_safe(const PackedProtocolFrame *frame) { uint32_t ts; memcpy(&ts, &frame->timestamp, sizeof(ts)); // safe: any alignment uint16_t len; memcpy(&len, &frame->length, sizeof(len)); // safe: any alignment (void)ts; (void)len; } // ── SAFER: avoid packed altogether ──────────────────────────── // Use explicit serialization/deserialization functions. // The struct has natural alignment; serialize to byte buffer when needed. typedef struct { uint8_t type; uint32_t timestamp; // natural alignment: offset 4 uint16_t length; // natural alignment: offset 8 uint8_t payload[8]; // offset 10 } AlignedFrame; // Serialize to network buffer (byte order and alignment explicit). void serialize_frame(const AlignedFrame *frame, uint8_t *buf) { buf[0] = frame->type; buf[1] = (uint8_t)(frame->timestamp >> 24); // big-endian buf[2] = (uint8_t)(frame->timestamp >> 16); buf[3] = (uint8_t)(frame->timestamp >> 8); buf[4] = (uint8_t)(frame->timestamp); buf[5] = (uint8_t)(frame->length >> 8); buf[6] = (uint8_t)(frame->length); memcpy(buf + 7, frame->payload, 8U); }

⚠ Never Use __attribute__((packed)) for Hardware Register Maps Hardware registers are at fixed addresses. The hardware itself enforces alignment: register addresses are always aligned to their natural width. Using __attribute__((packed)) on an MMIO struct removes the compiler's alignment guarantees. The struct may then map the wrong bytes to registers if any members are misaligned. Use __attribute__((packed)) only for protocol frame buffers that are serialized/deserialized explicitly, never for MMIO structs. On ARM targets: always verify packed struct behavior with a unit test that checks field values after a known write, or avoid packed entirely.

## 6.6  Designated Initializers: Safe C17 Initialization

Designated initializers, introduced in C99 and refined in C17, allow struct members to be initialized by name rather than by position. They are one of the most valuable safety features in C for struct initialization because they make initialization self-documenting, order-independent, and resistant to errors when struct members are added or reordered.

Listing 6.8 — designated initializers: safety advantages

#include <stdint.h> #include <stdbool.h> typedef struct { uint32_t device_id; float sample_rate_hz; uint16_t resolution_bits; bool differential_mode; uint8_t channel_mask; uint32_t timeout_ms; } AdcConfig; // ── UNSAFE: positional initialization ───────────────────────── // Any reordering of struct members silently changes meaning. // Adding a new member in the middle breaks all initializers. AdcConfig adc_UNSAFE = { 42U, // device_id 1000.0f, // sample_rate_hz 12U, // resolution_bits true, // differential_mode 0x0FU, // channel_mask 500U // timeout_ms }; // ── SAFE: designated initializer ────────────────────────────── // Member names are explicit: order doesn't matter. // Adding new members does not break existing initializers. // Uninitialized members are zero-initialized automatically. AdcConfig adc_SAFE = { .device_id = 42U, .sample_rate_hz = 1000.0f, .resolution_bits = 12U, .differential_mode = true, .channel_mask = 0x0FU, .timeout_ms = 500U, }; // ── Partial initialization: unspecified members are zeroed ───── // C17 guarantees: members not mentioned in a designated // initializer are zero-initialized (for static) or // value-initialized (equivalent) for local variables. AdcConfig adc_minimal = { .device_id = 1U, .sample_rate_hz = 100.0f, // All other members: zero-initialized. // resolution_bits = 0, differential_mode = false, // channel_mask = 0, timeout_ms = 0. }; // ── Compound literal with designated initializer ────────────── // C99/C17: compound literals allow creating struct values inline. // Useful for passing struct arguments without a named variable. void configure_adc(const AdcConfig *cfg); void init_sensors(void) { configure_adc(&(AdcConfig){ .device_id = 2U, .sample_rate_hz = 500.0f, .channel_mask = 0xFFU, .timeout_ms = 100U, }); } // ── Array designated initializers ───────────────────────────── // Also works for arrays: initialize specific indices by name. static const uint32_t PRIORITY_TABLE[8] = { [0] = 10U, // lowest priority [3] = 50U, // medium priority [7] = 90U, // highest priority // indices 1,2,4,5,6: zero-initialized };

## 6.7  Flexible Array Members

A flexible array member is a nameless array of unspecified size as the last member of a struct. Introduced in C99, it allows a struct to be allocated with variable-length data appended to the end. The struct declares the fixed-size header, and memory is allocated with enough space for the header plus the desired array length.

Listing 6.9 — flexible array members: correct use and constraints

#include <stdint.h> #include <stdlib.h> #include <string.h> #include <assert.h> // ── Flexible array member: variable-length token sequence ───── // The struct has a fixed header and a variable-length token array. typedef struct { uint32_t token_count; // number of tokens uint32_t capacity; // allocated capacity uint32_t source_id; // which input produced these tokens int32_t tokens[]; // flexible array: actual token IDs // sizeof(TokenSequence) does NOT include the flexible array. // sizeof(TokenSequence) == 12 on typical platforms. } TokenSequence; static_assert(sizeof(TokenSequence) == 12U, "TokenSequence header must be 12 bytes"); // ── Allocation: sizeof + array ──────────────────────────────── // Must allocate sizeof(struct) + n * sizeof(element_type). TokenSequence *token_seq_alloc(uint32_t capacity) { if (capacity == 0U ¦¦ capacity > 65536U) return NULL; size_t total = sizeof(TokenSequence) + (size_t)capacity * sizeof(int32_t); TokenSequence *seq = (TokenSequence *)malloc(total); if (seq == NULL) return NULL; seq->token_count = 0U; seq->capacity = capacity; seq->source_id = 0U; memset(seq->tokens, 0, capacity * sizeof(int32_t)); return seq; } void token_seq_free(TokenSequence *seq) { free(seq); } bool token_seq_append(TokenSequence *seq, int32_t token_id) { if (seq == NULL) return false; if (seq->token_count >= seq->capacity) return false; seq->tokens[seq->token_count++] = token_id; return true; } // ── Constraints on flexible array members ───────────────────── // 1. Must be the LAST member of the struct. // 2. The struct must have at least one other member. // 3. Cannot be a member of a union. // 4. sizeof() does not include the flexible array. // 5. Cannot be statically allocated: always requires malloc. // (Static: TokenSequence s; gives array of size 0.) // ── MISRA note: flexible arrays are restricted in MISRA C:2012 ─ // MISRA C:2012 Rule 18.7 (Advisory): flexible array members // should not be used. The rationale: sizeof does not reflect // total allocated size, which complicates static analysis. // In safety-critical code: prefer a fixed-size array with a // separately tracked length for MISRA compliance. typedef struct { uint32_t token_count; uint32_t capacity; uint32_t source_id; int32_t tokens[256]; // MISRA-compliant: fixed size } TokenSequenceFixed; // sizeof() is complete and accurate

## 6.8  Nested Structs, Unions, and Anonymous Members

Nested structs and unions are essential tools for organizing complex hardware register layouts and protocol data structures. C11 introduced anonymous structs and unions, which provide a cleaner syntax for accessing nested members without an intermediate member name.

### 6.8.1  Unions for Type-Safe Overlays

A union is a type whose members all share the same storage. The union's size is the size of its largest member, and all members start at the same address. Unions are used for: viewing the same bytes as different types (type punning), implementing discriminated unions (tagged unions), and mapping register fields at the same address with different interpretations.

Listing 6.10 — unions: type punning, tagged unions, register overlays

#include <stdint.h> #include <stdbool.h> // ── Pattern 1: Type punning via union (C99/C11 defined) ──────── // Reading a different member than was last written is defined // behavior in C (via the 'common initial sequence' rule and // C99 explicit allowance for union type punning). // Note: NOT defined in C++; use memcpy for C++ type punning. typedef union { float f; uint32_t u; } FloatBits; uint32_t float_to_bits_C(float value) { FloatBits fb; fb.f = value; return fb.u; // defined in C: union type punning } // ── Pattern 2: Register overlay union ───────────────────────── // Hardware register that can be accessed as a whole word // or as individual byte fields. typedef union { uint32_t word; // whole register struct { // byte access (C11 anonymous struct) uint8_t byte0; // bits 7:0 uint8_t byte1; // bits 15:8 uint8_t byte2; // bits 23:16 uint8_t byte3; // bits 31:24 }; } RegisterWord; // Usage: read whole word, then extract bytes without casting. void process_register(volatile const uint32_t *reg_addr) { RegisterWord rw; rw.word = *reg_addr; // read hardware register uint8_t status = rw.byte0; // extract byte 0 (clean, no UB) uint8_t config = rw.byte1; // extract byte 1 (void)status; (void)config; } // ── Pattern 3: Tagged union (discriminated union) ───────────── // Common in protocol parsers and message dispatchers. typedef enum { MSG_SENSOR = 1U, MSG_ACTUATOR = 2U, MSG_STATUS = 3U, } MessageType; typedef struct { uint32_t sensor_id; float value; } SensorMsg; typedef struct { uint32_t actuator_id; int32_t command; } ActuatorMsg; typedef struct { uint32_t error_code; uint32_t timestamp; } StatusMsg; typedef struct { MessageType type; // discriminant: which union member is valid union { SensorMsg sensor; ActuatorMsg actuator; StatusMsg status; } payload; } Message; // Safe dispatch: check type before accessing payload. void dispatch_message(const Message *msg) { switch (msg->type) { case MSG_SENSOR: handle_sensor(&msg->payload.sensor); break; case MSG_ACTUATOR: handle_actuator(&msg->payload.actuator); break; case MSG_STATUS: handle_status(&msg->payload.status); break; default: log_unknown_message_type(msg->type); break; } }

### 6.8.2  Anonymous Structs and Unions (C11)

C11 introduced anonymous struct and union members: nested structs or unions declared without a member name. Members of anonymous structs and unions are accessed directly as if they were members of the enclosing struct or union. This provides a cleaner API for complex register layouts and eliminates the intermediate member name that would otherwise be required.

Listing 6.11 — anonymous structs and unions

#include <stdint.h> // ── Without anonymous union: verbose access ─────────────────── typedef struct { uint32_t id; union { float f_value; // union member name required int32_t i_value; } data; // struct member name required } NonAnonymous; // Access: must use both member names. NonAnonymous na; na.id = 1U; na.data.f_value = 3.14f; // verbose: na.data.f_value // ── With anonymous union: direct access (C11) ───────────────── typedef struct { uint32_t id; union { // anonymous: no union member name float f_value; // accessed directly int32_t i_value; // accessed directly }; // semicolon, no name } Anonymous; // Access: direct, without intermediate member name. Anonymous a; a.id = 1U; a.f_value = 3.14f; // cleaner: a.f_value (not a.data.f_value) // ── Practical example: SPI register with bit-field and word access ─ typedef union { volatile uint32_t word; // whole-register access struct { // anonymous: direct field access volatile uint32_t CPOL : 1; // clock polarity volatile uint32_t CPHA : 1; // clock phase volatile uint32_t BR : 3; // baud rate divisor volatile uint32_t SPE : 1; // SPI enable volatile uint32_t LSBFIRST: 1; // LSB transmitted first volatile uint32_t SSI : 1; // internal slave select volatile uint32_t SSM : 1; // software slave management volatile uint32_t RXONLY : 1; // receive only volatile uint32_t DFF : 1; // data frame format volatile uint32_t CRCNEXT : 1; // transmit CRC next volatile uint32_t CRCEN : 1; // hardware CRC enable volatile uint32_t BIDIOE : 1; // bidirectional data output volatile uint32_t BIDIMODE: 1; // bidirectional data mode volatile uint32_t : 18; // reserved }; } SPI_CR1; // Usage: clean access to both whole register and individual fields. // WARNING: bit-field layout is still implementation-defined. // Add static_assert on the register word values to verify. void spi_enable(SPI_CR1 *cr1) { cr1->SPE = 1U; // direct field access (not cr1->bits.SPE) } uint32_t spi_get_control_word(const SPI_CR1 *cr1) { return cr1->word; // whole-register access }

## 6.9  Safety Standards and MISRA on Struct Layout

ISO 26262 and the MISRA C:2012 guidelines have specific requirements for struct usage in safety-critical code. These requirements are driven by the certification principle that software behavior must be fully predictable and analyzable. Implementation-defined struct layout behavior undermines this predictability.

📋 ISO 26262 — Part 6, Clause 8.4.4 — Language Constructs for C/C++ ISO 26262 requires that the use of implementation-defined behavior be documented in the software's configuration management records. For struct layout, this means: all implementation-defined properties (member sizes, alignment, padding amounts) must be documented and verified for the specific compiler and target platform combination. The standard specifically cites struct layout as a potential source of implementation-defined behavior that must be analyzed. The analysis must demonstrate that the layout is correct for the target hardware and that any changes to the compiler or platform would be detected (e.g., by static_assert checks). At ASIL C and D: every struct that interfaces with hardware, is shared across processes, or is transmitted over a communication bus must have its layout verified with compile-time assertions (static_assert) on every member's offset and the total struct size.

📘 MISRA — MISRA C:2012 Rule 19.2 — Advisory The union keyword should not be used. The rationale: union member access is inherently type-unsafe. Reading a member other than the one most recently written is implementation-defined behavior for non-character types (except in C99/C11 which explicitly allow union type punning). In safety-critical code: if unions are used (with documented justification), always use a tagged union with an explicit discriminant, and always verify the discriminant before accessing any payload member. Never read a union member whose type differs from the most recently written member except through a character or byte type.

📘 MISRA — MISRA C:2012 Rule 19.1 — Mandatory An object shall not be assigned or copied to an overlapping object. This rule prevents undefined behavior from overlapping memory copies. For struct copies, it means: never copy a struct into a region of memory that overlaps with the source. Use memcpy (which has defined behavior for non-overlapping regions) or memmove (which handles overlap correctly) rather than struct assignment when memory regions may overlap.

| MISRA C:2012 Rule | Classification | Struct/Union Topic |
| --- | --- | --- |
| Rule 6.1 | Required | Bit-fields: only explicitly typed unsigned or signed int |
| Rule 6.2 | Required | Bit-fields: single-bit fields must be unsigned |
| Rule 18.7 | Advisory | Flexible array members should not be used |
| Rule 19.1 | Mandatory | No overlapping struct copy |
| Rule 19.2 | Advisory | Union keyword should not be used |
| Rule 21.2 | Required | Macro names shall not be declared reserved identifiers |
| Implied by 1.3 | Mandatory | No use of implementation-defined padding bytes |
| Implied by 8.4.4 | Mandatory | All IDB struct layout must be documented and verified |

## 6.10  LLM Angle: Vocabulary Structs, Tensor Descriptors, KV Cache Layout

LLM inference systems in C++ make heavy use of structs for organizing the key data structures of the inference pipeline. The same principles of layout discipline that govern safety-critical hardware register mapping apply directly to the data structures of tokenization and inference: incorrect alignment wastes memory, incorrect padding corrupts data, and unverified layouts produce silent bugs.

### 6.10.1  Vocabulary Entry Layout

The tokenizer's vocabulary stores a mapping between string tokens and integer IDs. The data structures for this mapping are accessed billions of times per inference run. Cache-friendly layout is critical for performance; layout correctness is critical for determinism.

Listing 6.12 — vocabulary data structures with layout discipline

#include <cstdint> #include <cstddef> #include <cassert> #include <string_view> #include <vector> // ── Vocabulary entry: fixed-size for cache line packing ──────── // Each entry should fit neatly into cache lines (64 bytes). // Short token strings stored inline; longer tokens use an index // into a separate string table. // Small-string-optimized token: tokens <= 15 bytes stored inline. struct alignas(16) TokenEntry { int32_t id; // offset 0: token ID uint8_t length; // offset 4: string length (0..15 inline) uint8_t token_type; // offset 5: 0=normal,1=special,2=byte uint16_t reserved; // offset 6: for future use char inline_str[8]; // offset 8: inline string storage (8 bytes) // Total: 16 bytes. Aligned to 16. 4 entries per cache line. }; static_assert(sizeof(TokenEntry) == 16, "TokenEntry must be exactly 16 bytes for cache packing"); static_assert(alignof(TokenEntry) == 16, "TokenEntry must be 16-byte aligned"); static_assert(offsetof(TokenEntry, id) == 0, "id at 0"); static_assert(offsetof(TokenEntry, length) == 4, "length at 4"); static_assert(offsetof(TokenEntry, inline_str) == 8, "str at 8"); // ── BPE merge rule entry: compact representation ────────────── // Each merge rule: (left_id, right_id) -> merged_id, with rank. // Packed tightly: no padding needed (all uint32_t). struct MergeRule { int32_t left; // offset 0 int32_t right; // offset 4 int32_t result; // offset 8 int32_t rank; // offset 12 // sizeof = 16, alignof = 4. No padding. }; static_assert(sizeof(MergeRule) == 16, "MergeRule must be 16 bytes"); static_assert(offsetof(MergeRule, left) == 0, "left at 0"); static_assert(offsetof(MergeRule, result) == 8, "result at 8"); static_assert(offsetof(MergeRule, rank) == 12, "rank at 12");

### 6.10.2  Tensor Descriptor Layout

A tensor descriptor is a metadata structure that describes a tensor's shape, strides, data pointer, and type without owning the tensor's data. Correct layout is critical: the descriptor is passed between functions, stored in arrays, and accessed on every tensor operation. Poor layout wastes cache lines; incorrect layout silently misidentifies tensor dimensions.

Listing 6.13 — tensor descriptor with verified layout

#include <cstdint> #include <cstddef> enum class DType : uint8_t { Float32 = 0, Float16 = 1, Int8 = 2, Int32 = 3, }; // Tensor descriptor: describes a tensor without owning its data. // Fixed at 48 bytes: fits in a single cache line (64 bytes) with // room for a pointer and a couple of control bytes alongside. struct alignas(8) TensorDesc { float *data; // offset 0: data pointer (8 bytes) int32_t shape[4]; // offset 8: shape dims [batch,seq,head,dim] int32_t strides[4]; // offset 24: strides in elements uint32_t ndim; // offset 40: number of valid dimensions DType dtype; // offset 44: data type uint8_t device; // offset 45: 0=CPU, 1=GPU uint8_t flags; // offset 46: bit flags uint8_t pad; // offset 47: explicit padding // Total: 48 bytes. alignof = 8. }; static_assert(sizeof(TensorDesc) == 48, "TensorDesc must be exactly 48 bytes"); static_assert(offsetof(TensorDesc, data) == 0, "data at 0"); static_assert(offsetof(TensorDesc, shape) == 8, "shape at 8"); static_assert(offsetof(TensorDesc, strides) == 24, "strides at 24"); static_assert(offsetof(TensorDesc, ndim) == 40, "ndim at 40"); static_assert(offsetof(TensorDesc, dtype) == 44, "dtype at 44"); // Safe dimension access with bounds check. inline int32_t tensor_dim(const TensorDesc *t, uint32_t i) { if (i >= t->ndim ¦¦ i >= 4U) return 0; return t->shape[i]; } inline int64_t tensor_numel(const TensorDesc *t) { int64_t n = 1; for (uint32_t i = 0U; i < t->ndim && i < 4U; i++) n *= t->shape[i]; return n; }

### 6.10.3  KV Cache Entry Layout for Cache Efficiency

The key-value cache in transformer inference stores floating-point vectors for each past token position across all layers and attention heads. Its layout has direct impact on inference latency: poor layout causes cache line splits on every attention computation.

Listing 6.14 — KV cache entry layout analysis

#include <cstdint> #include <cstddef> // KV cache for a single attention head, single layer, single position. // head_dim typically 64 or 128 floats. // Keys and values stored together for spatial locality during attention. // ── Layout option 1: Separate key and value arrays ──────────── // key_cache[layer][head][pos][dim] and val_cache[layer][head][pos][dim] // Pro: can load all keys for one layer without interleaving values. // Con: two separate allocations; pointer chasing in attention. // ── Layout option 2: Interleaved key-value per position ──────── // For each position: [key_0, key_1,..., key_d, val_0, val_1,..., val_d] // Pro: attention reads K and V for the same position are adjacent. // Con: loading only keys for a full sequence skips every other block. // ── Layout option 3 (common in production): layer-major ──────── // Outer dim: layer. Inner dims: position, head, dim. // Accessing all heads for one layer+position is sequential. // Compact KV entry: both key and value in one struct. // head_dim = 64 floats. Two vectors = 512 bytes = 8 cache lines. template<int HEAD_DIM> struct alignas(64) KVEntry { float keys[HEAD_DIM]; // key vector: HEAD_DIM floats float values[HEAD_DIM]; // value vector: HEAD_DIM floats // Total: 2 * HEAD_DIM * 4 bytes // For HEAD_DIM=64: 512 bytes = 8 cache lines }; using KVEntry64 = KVEntry<64>; static_assert(sizeof(KVEntry64) == 512, "KVEntry64 must be 512 bytes (2 * 64 floats)"); static_assert(alignof(KVEntry64) == 64, "KVEntry64 must be cache-line aligned"); // KV cache for one layer: [max_seq_len][num_heads] of KVEntry. struct KVCacheLayer { uint32_t max_seq_len; uint32_t num_heads; uint32_t current_len; uint32_t pad; KVEntry64 entries[]; // flexible: max_seq_len * num_heads entries }; static_assert(sizeof(KVCacheLayer) == 16, "KVCacheLayer header must be 16 bytes");

🧠 LLM Angle Layout discipline for LLM data structures is not merely a performance concern — it is a correctness concern. A TensorDesc with wrong stride values produces incorrect attention scores silently. A KV cache with wrong padding between layers causes one layer's keys to be read as another layer's values. The static_assert discipline applied to hardware register structs applies identically to LLM data structures. Every struct that crosses a module boundary (tokenizer to inference engine, inference engine to sampler) should have its layout verified with compile-time assertions. Cache-line alignment of KV cache entries is critical for GPU performance. An unaligned 512-byte KV entry causes split cache line transactions on every attention read. On modern NVIDIA GPUs, this can reduce attention throughput by 20-30%. For safety-adjacent LLM deployments: treat TensorDesc, KVEntry, and TokenEntry layout verification with the same rigor as MMIO register layout verification. A wrong tensor dimension read produces wrong model output as silently as a wrong register read produces wrong hardware behavior.

## Chapter Exercises

Exercise 6.1 — Struct Layout Prediction and Verification Before compiling, predict the sizeof and all member offsets for each of these structs: (a) struct { uint8_t a; uint64_t b; uint8_t c; }; (b) struct { uint32_t x; uint16_t y; uint8_t z; uint8_t w; }; (c) struct { double d; uint8_t b; uint32_t i; uint16_t s; }; Verify each prediction with offsetof() and sizeof(). Write static_assert statements documenting each. Now reorder each struct's members from largest to smallest alignment. How much does sizeof() change?

Exercise 6.2 — MMIO Struct Mapping Take the UART_TypeDef from Listing 6.4 and verify all offsets compile correctly on your platform. Add a function uart_set_baud(UART_TypeDef *uart, uint32_t target_baud, uint32_t clock_hz) that computes and sets IBRD and FBRD correctly. Write a unit test that initializes the struct with known values, reads them back, and verifies byte-exact correctness. Research: what does the FR register's TXFE bit mean? When should you check it vs TXFF?

Exercise 6.3 — Packed Struct Portability Create a packed struct with a mix of uint8_t, uint16_t, and uint32_t members. Write a function that accesses each member using direct member access (->). Compile for ARM Cortex-M (using arm-none-eabi-gcc or QEMU) with -march=armv7-m. Does accessing the misaligned members cause a HardFault? Check the Cortex-M CCR register setting. Rewrite the access functions using memcpy as shown in Listing 6.7. Does the HardFault disappear?

Exercise 6.4 — LLM Data Structure Layout Audit Take the TensorDesc from Listing 6.13. Add a fifth dimension to shape[] and strides[]. What is the new sizeof(TensorDesc)? Is it still 48 bytes? Update the static_asserts. Design a KV cache layout for a 32-layer model with 32 heads per layer, head_dim=128, max_seq_len=4096. Calculate: how many bytes of KV cache does one inference request require? How does this compare to the model's weight size (assuming FP16, 7B parameters)?

Chapter 6 — Key Takeaways - Struct layout is determined by a precise algorithm: members are placed in declaration order with compiler-inserted padding to satisfy each member's alignment requirement, and trailing padding is added so sizeof() is a multiple of the struct's alignment. - The alignment of a struct equals the maximum alignment of any of its members. Nested structs with high-alignment members (double, pointer) impose that alignment on the enclosing struct. - offsetof() and sizeof() are the primary tools for documenting and verifying struct layout. static_assert must be used to verify the layout of every hardware-mapped struct, protocol buffer struct, and inter-process communication struct at compile time. - MMIO struct mapping requires: all members declared volatile, all reserved registers included as padding members, all offsets verified with static_assert, and the base address bound as a compile-time constant pointer. - Bit-fields provide named access to hardware register fields but have implementation-defined layout. For safety-critical hardware access, prefer explicit masks and shifts over bit-fields. When bit-fields are used, declare them with explicit uint8_t/uint16_t/uint32_t types (MISRA C:2012 Rule 6.1). - __attribute__((packed)) eliminates padding and is useful for protocol frames but causes misaligned access on ARM Cortex-M. All accesses to packed struct fields must use memcpy. Never use packed structs for MMIO register maps. - Designated initializers (.member = value) are the safe default for struct initialization in C17: they are order-independent, self-documenting, and zero-initialize unmentioned members. Positional initializers are fragile and must not be used in safety-critical code. - Flexible array members are restricted in MISRA C:2012 (Rule 18.7 Advisory) because sizeof() does not reflect total allocated size. Use fixed-size arrays with separately tracked lengths for MISRA compliance. - LLM data structures (TokenEntry, TensorDesc, KVEntry) must receive the same layout discipline as hardware register structs: static_assert on every member offset, cache-line alignment for performance-critical structures, and designation of every padding byte.